SUMMARY

Water is an essential component for survival of life on earth, which contains minerals, important for humans as well as for earth and aquatic life. Lakes and surface water reservoirs are the planet’s most important freshwater resources and provide innumerable benefits. They are used for domestic and irrigation purposes, and provide ecosystems for aquatic life especially fish, thereby functioning as a source of essential protein, and for significant elements of the world’s biological diversity. Water is one of the important sources, to sustain life and has long been suspected of being the source of much human illness. Source of surface water and ground water have become increasingly contaminated due to increased industrial and agricultural activity. The quality of surface water is largely affected by natural processes (weathering and soil erosion) as well as anthropogenic inputs (municipal and industrial wastewater discharge). The anthropogenic discharges represent a constant polluting source, whereas surface run off is a seasonal phenomenon, largely affected by climatic conditions.

The public desires water that is low in hardness and total solids, non- corrosive and non- scale forming. To provide such water, chemists, biologists and engineers must combine their efforts and talents. Chemists, through their knowledge of colloidal, physical and organic chemistry are especially helpful in solving problems related to the removal of colour, turbidity, hardness, harmful metal ions and organic compounds and to the control of corrosion and scaling. The biologists are often of great help in taste and odour problems that derive from aquatic growths. The present study was carried out water quality of Madiwala lake, From September 30/2024 to December 21 /2024 the water quality survey of Madiwala Lake was conducted. The study provided the much-needed baseline data to review the problems associated with the water quality.

ABSTRACT

Water is the most important in shaping the land and regulating the climate. It is one of the most important compounds that profoundly influence life. The quality of water usually described according to its physical and chemical characteristics. Rapid industrialization and indiscriminate use of chemical fertilizers and pesticides in agriculture are causing heavy and varied pollution in aquatic environment leading to deterioration of water quality and depletion of aquatic biota. Due to use of contaminated water, human population suffers from water borne diseases. It is therefore necessary to check the water quality at regular interval of time. Parameters that may be tested include taste, odour, pH, nitrates and alkalinity. An assessment of the aquatic macro invertebrates can also provide an indication of water quality.

KEYWORDS: Alkalinity, Dissolved Oxygen, Acidity, Chloride, pH, Total hardness, Nitrates

CONTENTS

ACKNOWLEDGEMENT

SUMMARY ……………………………………………………………             	I

ABSTRACT……………………………………………….………………   	II

CHAPTER …………………………………………………………………….

LIST OF FIGURES

LIST OF TABLES

Pg No.

CHAPTER 1: INTRODUCTIO                                                                               1-3

1.1 INTRODUCTION

1.2 DRINKING WATER

1.3 DISTRIBUTION OF WATER ON EARTH

1.4 STUDY AREA

1.5 AIM AND OBJECTIVE OF STUDY

CHAPTER 2 LITERATURE REVIEW                                                                    4-9

CHAPTER 3 OBJECTIVES OF WATER QUALITY ANALYSIS                          10

CHAPTER 4 METHODOLOGY                                                                            11-14

CHAPTER 5 DETERMINATION OF SAMPLE PARAMETERS                        15-29

5.1 pH

5.2 Alkalinity

5.3 Acidity

5.4 Total Hardness

5.5 Total Solids

CHAPTER 6

6.1 RESULT                                                                                         30

6.2 CONCLUSION                                                                            31 -32

REFERENCE                                                                                                            33

LIST OF FIGURES

| FIGURE NO | TITLE PAGE NO | PAGE NO |
| --- | --- | --- |
| Figure 1.1 | Distribution of Water | 2 |
| Figure 1.2 | Madiwala lake | 3 |
| Figure 4.1 | Block Diagram of Water purification | 11 |
| Figure 5.1 | pH scale | 16 |
| Figure 5.2 | pH meter | 17 |
| Figure 5.3 | pH paper | 18 |
| Figure 5.4 | Ending point of phenolphthalein alkalinity | 19 |
| Figure 5.5 | Start point of methyl orange alkalinity | 19 |
| Figure5.6 | End point of methyl orange alkalinity | 20 |
| Figure 5.7 | Starting of C02 Acidity | 22 |
| Figure 5.8 | End point of C02 Acidity | 22 |
| Figure 5.9 | End point of mineral Acidity | 24 |
| Figure 5.10 | Starting point Total hardness | 25 |
| Figure 5.11 | End point of Total hardness | 25 |
| Figure 5.12 | Starting point Calcium hardness | 26 |
| Figure 5.13 | End point of Calcium hardness. | 26 |
| Figure 5.14 | Total Dissolved Solids | 27 |

LIST OF TABLES

| TABLE NO | TITLE | PAGE NO |
| --- | --- | --- |
| Table 5.1 | Result of pH | 18 |
| Table 5.2 | Result of Alkalinity | 21 |
| Table 5.3 | Result of Acidity | 23 |
| Table 5.4 | Result of Total hardness | 27 |
| Table 6.1 | Results As per BIS for drinking water (IS 10500-2012 | 30 |